GENERAL SPECIFICATIONS OF

CONTAINER MODIFIED HOUSING (CMH)

| 1. | GENERAL | 1.a | 40’ containers shall be modified to make the units. |
| --- | --- | --- | --- |
|  |  | 1.b | Exterior of Container shall be re-painted with two coats of alkyd based enamel paint. |
| 2. | FLOOR | 2.a | Decking : 18mm commercial plywood, fixed to the container floor. |
|  |  | 2.b | Floor covering in dry areas: 2.0mm thick PVC vinyl tiles (30 x 30cm) / sheet fixed to plywood floor. Brand : Polyflor or equivalent. |
|  |  | 2.c | Floor covering in wet areas: 2.0mm thick PVC vinyl sheet fixed to plywood floor, joints heat welded for waterproofing. Brand : Polyflor –or equivalent. |
| 3. | WALLS | 3.a | External finishing: Container finish. |
|  |  | 3.b | Framing: Timber framing 45mm x 45mm at a spacing of 600mm C/C vertically. |
|  |  | 3.c | Internal finishing: 3mm thick décor ply fixed to wall framing. Finishing in wet areas shall be 3mm thick laminated tile board. |
|  |  | 3.d | Wall insulation: 45mm thick polystyrene 11-12 Kg/m3 density. |
| 4. | ROOF | 4.a | Roofing: Same as container roofing. |
| 5. | CEILING. | 5.a | Framework: 45mm x 45mm timber framework. |
|  |  | 5.b | Ceiling finishing: 4mm plywood finished with light texture, good quality interior paint . Ceiling clear height shall be 2.15m |
|  |  | 5.c | Insulation : 45mm polystyrene 11-12 Kg/m3 density. |

| 6. | DOORS | 6.a | External door: Steel door with GI door frame and sand wich panel shutter with steel plates on either sides and honey comb core. Size as per drawings. |
| --- | --- | --- | --- |
|  |  | 6.b | Door lock: "Kafli" brand, cylinder type for all doors and indicator latch for bathroom doors. |
| 7. | WINDOWS | 7.a | Silver anodized aluminum frame (Non thermal break), horizontal sliding window (one fixed, one sliding) with insect screen and weather stripping, 6mm thick clear glass(Single glazing), overall size 800mm x 1000mm . Venetian blinds shall be provided for all windows. |
|  |  | 7.b | Exhaust window : Fixed window with red miranti timber framing and 6mm thick obscure glass. |
| 8. | ELECTRICAL | 8.a | Electrical supply is 240V, 50 Hz, Single phase power supply as per requirement. |
|  |  | 8.b | Electrical Panel board is provided with ELCB incomer and MCB outgoing. Brand : MEM /BILL / Electra – GE or equ. |
|  |  | 8.c | PVC Conduits are concealed within the wall panels. |
|  |  | 8.d | Light fixtures: Ceiling bowl light shall be provided as per drawings. Brand: Sozzi, Italy or equivalent. 1X60W/P bulkhead light shall be supplied in loose which may be fixed later. |
|  |  | 8.e | Switch / sockets , Crabtree / MEM or equivalent 1 Nos of Industrial socket shall be provided externally to connect power from external sources |
|  |  | 8.f | Water heater, 12 US Gl capacity, Thermex or equivalent provided as per the drawings |
| 9. | HVAC | 9.a | Exhaust fans are provided for toilet as shown in drawings. (KDK – Japan, 6"dia window mounted type). |
|  |  | 9.b | A/c openings are provided with A/c brackets fabricated from MS angles. |

| 10. | PLUMBING | 10.a. | All cold water lines and sewage lines are PVC pipes. Hot water lines are copper pipes. |
| --- | --- | --- | --- |
|  |  | 10.b | Sanitary wares : Wash hand basins will be pedestal type (RAK Ceramics. Liwa range) as per drawings. EWC with cistern (RAK Ceramics. Tampa range) Ablution hose and toilet paper holder as per drawings. GRP shower cubicle / GRP shower tray as per the drawings. |
|  |  | 10.c | Sanitary fittings: Mixers provided from Bonus or equivalent. PVC soap tray, Towel rod, coat hook etc shall be provided. |
| 11. | BUILT IN WARD ROBE | 11.a | Built in ward robe made of plywood with laminated finish and door made of MDF shall be provided as per drawings. |
| 12. | COMPUTER POINT | 12.a | PVC conduits, cables, RJ sockets and jack and networking shall be provided as per drawings. A 10 pair termination block (KRONE – 71) shall be installed inside 300x300x250 mm GI junction box for each caravan. |